### Lesson 12 Teacher Reference

# Setup for Collision Carts Demonstration

| - Add textbooks or binders to elevate one end of the flooring strip 8”-12” to create a ramp. |  |
| --- | --- |
| - Angle the lower end of the vinyl flooring strip into the brick. |  |
| - Place the metal track on the vinyl flooring strip and with its lower end against the brick. - Place the push-pull spring scales on the carts using the hook and loop fasteners. |  |
| - Mount a plastic, blank electrical plate on the end of the rod on each of the 2 push-pull spring scales using sticky tack. |  |
| - Make sure that the two electrical plates touch flush when the carts are pushed together. |  |
| - Using duct tape, tape down one cart at the bottom of the ramp about 4 inches away from the brick with its plate facing up the ramp. |  |