# Lesson 14: How can we use our science ideas and other societal wants and needs to refine our designs?

| Previous Lesson | We developed a model to represent how the structures of materials compare in the top four performers for peak force reduction. We used scaled-up versions of these structures to generate data using slow-motion video about the unobservable mechanisms at work in the system. We carried out an investigation to determine how the amount of force applied to different points of a cushioning structure is affected by the shape of that structure. |  |
| --- | --- | --- |
| This Lesson Putting Pieces Together 2 days |  | We redesign our device using our science ideas and consider the implications of those changes. We share our redesigns with stakeholders to get new perspectives and feedback. We use the new stakeholder considerations to evaluate the potential design materials and use weighted considerations to analyze the choices. We reflect on the consequences of those potential changes based upon stakeholder feedback. |
| Next Lesson | We will evaluate other engineers’ design solutions to protect cheerleaders from concussions in collisions using the science and engineering ideas we have figured out over the course of the unit. We will design our own solution and argue how it takes into consideration the criteria, constraints, and trade-offs in the proposed solution. We will revisit the DQB to take stock of the questions we have answered. |  |

| BUILDING TOWARD NGSS MS-PS2-1, MS-PS2-2, MS-PS3-1, MS-ETS1-2, MS-ETS1-3, MS-LS1-8 | What students will do 14.A Optimize the performance of a design by prioritizing the particular functions and properties of materials based upon stakeholder feedback to assess the relative effectiveness of the materials. 14. B Engage in a quantitative analysis using prioritized scores and consider the trade-offs of prioritizing particular uses and functions of materials to assess their relative effectiveness for the optimization of the design. What students will figure out - Most designs will not meet every criterion and constraint perfectly. - When engaging in an engineering design problem, trade-offs will occur based upon stakeholder feedback. - Some stakeholder considerations will have a higher priority than other considerations. - Every trade-off has consequences. |
| --- | --- |

### Lesson 14 • Learning Plan Snapshot

| Part | Duration |  | Summary | Slide | Materials |
| --- | --- | --- | --- | --- | --- |
| 1 | 5 min |  | REVISIT MATERIALS TESTING RESULTS Look back at what we have learned about material properties and structures. | A |  |
| 2 | 15 min |  | REVISE DESIGNS Revise designs using protective material science ideas. | B-F | Drafting Our Protection Device Design from Lesson 11, Redesigning Your Protective Device |
| 3 | 5 min |  | UPDATE CRITERIA AND REVISIT CONSTRAINTS Look back at the Whole-Class Criteria and Constraints chart from Lesson 11. Refine what it means to be a protective material in criteria, and revisit constraints. |  | Whole-Class Criteria and Constraints chart from Lesson 11 |
| 4 | 10 min |  | DISCUSS TRADE-OFFS USING A COMMON EXAMPLE Look at a commonly used protective device to analyze potential trade-off decisions made by engineers and determine that even professionals have to make trade-offs. | G-H |  |
| 5 | 8 min |  | HOME LEARNING Go over Stakeholder Feedback Form and plan to receive stakeholder feedback. | I-J | Redesigning Your Protective Device, Stakeholder Feedback Form |
| ##### End of day 1 |  |  |  |  |  |
| 6 | 5 min |  | SHARE FEEDBACK FROM HOME LEARNING Learn about market research and share stakeholder feedback with a partner. | K-L | Redesigning Your Protective Device, Stakeholder Feedback Form |
| 7 | 15 min |  | UTILIZE A DECISION MATRIX Use a decision matrix to compare material trade-offs for the protective device designs. | M-P | Redesigning Your Protective Device, Stakeholder Feedback Form, Decision Matrix, Additional Material Considerations Matrix |
| 8 | 15 min |  | USE TRADE-OFF THINKING TO ANALYZE DESIGNS Determine primary, secondary, and tertiary rankings of criteria. Use those rankings to alter the overall score of each material. | Q-U | Stakeholder Feedback Form, Decision Matrix, Additional Material Considerations Matrix, calculator (optional), 2 rulers (optional) |
| 9 | 8 min |  | CONSIDER CONSEQUENCES OF TRADE-OFFS Consider the potential consequences of changing materials or elevating certain criteria or constraints above others based upon stakeholder feedback. | V | Redesigning Your Protective Device, Decision Matrix |
| 10 | 2 min |  | NAVIGATION Look ahead to synthesizing information in an argument. |  |  |
| ##### End of day 2 |  |  |  |  |  |

### Lesson 14 • Materials List

|  | per student | per group | per class |
| --- | --- | --- | --- |
| Lesson materials | - science notebook - Drafting Our Protection Device Design from Lesson 11 - Redesigning Your Protective Device - Stakeholder Feedback Form - Decision Matrix - Additional Material Considerations Matrix - calculator (optional) - 2 rulers (optional) |  | - Whole-Class Criteria and Constraints chart from Lesson 11 |

## Materials preparation (15 minutes)

Review teacher guide, slides, and teacher references or keys (if applicable).

Make copies of handouts and ensure sufficient copies of student references, readings, and procedures are available.

Ensure all students have their copy of Drafting Our Protection Device Design from Lesson 11.

Get out and have ready to post the Whole-Class Criteria and Constraints chart from Lesson 11.

### Lesson 14 • Where We Are Going and NOT Going

#### Where We Are Going

Students will engage in the design revision process to quantitatively evaluate the optimization of materials for designs. Using the revised design, students will receive feedback from stakeholders on potential considerations and trade-offs from the stakeholders’ perspectives. Students will then engage in a systematic process of evaluating their materials for design solutions (ETS1.B; ETS1.C) and interpret the quantitative data produced from the ranking of stakeholder considerations based upon stakeholder feedback.

#### Where We Are NOT Going

While we may be prioritizing criteria and assigning them numerical scores dealing with the design trade-offs, we will not be sequencing the subproblems that need to be solved in order to systematically optimize the design. The consequences of each trade-off and prioritization will not be purposefully analyzed from a cultural standpoint, economic standpoint, or any other sources of stakeholder resistance to the solutions. Environmental impacts are listed for students to use as possible considerations in their materials selection, but no time will be spent analyzing whether the design solutions meet the stakeholders’ acceptable level of social and environmental impacts associated with those considerations.

# LEARNING PLAN for LESSON 14

| ### 1 · REVISIT MATERIALS TESTING RESULTS | 5 min |
| --- | --- |
| MATERIALS: science notebook |  |
| Recall previous learning. Display slide A. Say, Think back to last class. What did we learn about the role of shape, thickness, or properties of protective materials that we should be using in our designs? Give students time to turn and talk with a partner about the prompt on slide A. After a moment, ask students to share their ideas with the whole group. As students are sharing, listen for the following ideas to be discussed: - Protective materials should have space (or air) for the solid materials to deform into. - Protective materials should deform a lot when relatively weak contact forces are applied to them. - The outer surface of those materials should be placed in an area where they will make contact with the damaging, colliding object. - The inner surface of those materials should make contact with as large an area as possible on the parts of the body or object they are protecting. - The thicker the protective material is (not always more dense), the more it will reduce peak forces. If these ideas are not mentioned, ask students to look back at their diagrams from Lessons 12 and 13. Ask students what they notice about the materials that are contributing to a reduction in the peak forces on the object they are protecting. #### ADDITIONAL GUIDANCE; The ideas about the properties of a material that provide more protection and how these properties contribute to a reduction in peak force on an object will be utilized as students revise their designs. If some students need a more-concrete version of this list, consider writing these ideas down on a piece of chart paper or whiteboard in the front of the room to serve as an idea list for the redesign and potential new selection of materials for their protective devices.. Say, It seems like we have learned a lot about the properties and structures of protective materials! Let’s see if we can apply this to optimize our designs. |  |

| ### 2 · REVISE DESIGNS | 15 min |
| --- | --- |
| MATERIALS: Drafting Our Protection Device Design from Lesson 11, Redesigning Your Protective Device |  |
| Introduce Redesigning Your Protective Device. Display slide B. Instruct students to get out their handout Drafting Our Protection Device Design from Lesson 11. Explain that students will get a chance at revising their old design from Lesson 11 to reflect the current understanding of materials and how they help reduce peak forces in a collision. Using a fresh, new handout to record their protective device redesign will allow students to keep a record of their first iteration of their designs as they draft their new ideas. Distribute Redesigning Your Protective Device to students and go over it with them. Begin by pointing out the box where students will be drafting their redesigned protective devices. Make sure that students know they will be redrawing their ideas in this box, using their old designs on Drafting Our Protection Device Design as a starting point. Direct students to then look at the table on the next page. Explain that this table will walk students through considering what changes they are making and why. Column 1 asks students to list the changes that are being made, column 2 asks students to give their reasoning for the changes based upon their science ideas, and column 3 asks students to think about what effects those changes will have upon their system. Because this chart is new, tell students that we will do an example together based upon a design to protect a car side mirror in a parking lot. Display slide C. Point out that the chart on the slide is a copy of the chart on their handouts. Say, Imagine that I had designed a protective device to put onto the side mirrors of my car when I am parked in a parking lot to avoid scratches and bumps. Originally I had made a design that was a box of wood because I thought it would be really sturdy, but I didn’t think about the energy transferring through the wood. Now I have changed my design to be made of the same foam as earplugs. Display slide D. Point out to students how the change is documented in the first column. Say, Now I need to think about how I can justify this change using my science ideas. I made the change from wood to foam because foam deforms more and can be shaped to spread the impact out over a larger area when it is being deformed. The wood would get bumped and then would just bump right back into my mirror! Display slide E. Point out to students how this rationale is documented in the second column of the chart. Say, Finally, I have to think about the change on the overall structure of the device. There are positive and negative effects that would happen if I use foam as my main material instead of wood. I like the idea of foam better, but I think that making it out of foam will have an effect on how durable it is. I decreased how durable my design is since the wood would definitely hold up better over time. Also, foam just can’t be nailed together like wood. Because it needs special glue or something to create, it will also be a lot harder or take more time to create and sell. On the other hand, the foam will be easier to store since it can deform and probably fit under a seat in the car. Display slide F. Direct students to look at the documentation of these ideas in the third column. Tell students that they will be given time to work on this redesign. Explain that if students have made more than 3 major changes to their designs to focus the chart on the biggest changes made based upon their science ideas. Give students 10 minutes to work on their own to complete Redesigning Your Protective Device. |  |

| ### 3 · UPDATE CRITERIA AND REVISIT CONSTRAINTS | 5 min |
| --- | --- |
| MATERIALS: Whole-Class Criteria and Constraints chart from Lesson 11 |  |
| Bring the class back together. Display the Whole-Class Criteria and Constraints chart from Lesson 11. Recap that up until this point our designs have focused on what materials will best help our designs protect our objects. We have spent some time learning about the materials’ properties and thinking about how best to meet those criteria. Point out that while we have done a great job addressing what we mean by good shapes and protective materials and incorporating our science ideas into our design, we also said that we have constraints to consider. Say, When we started to design our devices, we created a Criteria and Constraints chart. Other than our criteria, what else did we say we needed to consider as a class? Allow students time to share out the items in the ”Constraints” column. If students would like to add more constraints than currently listed on the chart, add those to the chart at this time. Continue discussing the constraints list with students. Determine that different devices will have different constraints depending on the use of the object being protected and that all constraints may not be met perfectly for the practical use of the device. Example prompts and responses are located in the following table. Suggested prompt; Sample student response. This is a long list of constraints. Would every constraint apply to everyone’s individual protective device design?; No.. Interesting. Let’s look at our list. Which constraints may be design-specific?; (Accept all valid responses.) (Students may mention ideas like designs needing to be waterproof or having size constraints.). OK, let’s think about each of the constraints that do apply to your devices. Looking at the constraints that are important to your device, did your device meet those constraints perfectly?; My device met most of the constraints. My device met the criteria, but I had to prioritize my constraints. (The majority of students will say that their device did not meet every single criterion and constraint.). So what I am hearing is that our designs don’t seem to meet all the criteria and constraints, but we tried to make the best decisions possible for our devices. Do you think that other protective device designers have to make decisions about what to prioritize as well?; Yeah, different devices have different considerations, and some have too many considerations to make everyone happy. The number of and reasoning behind different considerations might make it hard for a single design to meet the criteria and constraints perfectly. Some people may have conflicting criteria and constraints, so both can’t be met perfectly. There has to be some compromise.. Say, It’s interesting that device designers seem to have to make the best decisions based upon the need and considerations of those that the device affects. It also sounds like some considerations are more important than others in most designs, and sometimes compromises need to be made. Do you think that a professionally designed item would also have some of these compromises in its design? Let’s take a look at a common protection device together to see if actual engineers have had to make the same kinds of decisions. |  |

| ### 4 · DISCUSS TRADE-OFFS USING A COMMON EXAMPLE | 10 min |
| --- | --- |
| MATERIALS: None |  |
| Discuss trade-offs with knee pads. Display slide G. Ask students to turn and talk about the questions on slide G. - What are some constraints that would apply to this protective device? - Are there any constraints that the design would have that are more important than others? Ask students to share their ideas with the class. Students should identify the constraints of size, cost, type of material used, shape, ease of use, reusability, and others as considerations that a designer would have to keep in mind when creating this device. Discuss trade-offs. Remind students that we learned about trade-offs in the Tsunami Unit and Homemade Heater Unit. Trade-offs are when criteria or constraints have to be weighed against each other to make an informed decision. When designers of these devices are weighing the options and the relative importance of criteria or constraints, they are making trade-offs. Ask students about the trade-offs made during the designing of the kneepad. Example prompts and responses are located in the following table. #### ADDITIONAL GUIDANCE; The term trade-off was utilized in Tsunami Unit to explain why students did not agree on the best choice for a tsunami-mitigation design. Trade-off thinking was discussed as weighing criteria and constraints against each other, and then those were used to determine which were more important to make an informed decision about what structure to utilize in a tsunami-mitigation design problem. The effects that these trade-offs had on other criteria, systems, or groups were also explored. Trade-offs were also explored in the creation of a device to heat an MRE in Homemade Heater Unit. In that unit, trade-offs were weighed to determine acceptable ratios of materials and their heating abilities. If students have not had exposure to the Tsunami Unit or Homemade Heater Unit, spend some time discussing how trade-offs can affect other aspects of the system, stakeholders, and other groups. The term trade-off can be added to the Word Wall if needed.. Suggested prompt; Sample student response. Looking at this kneepad device, what do you think are some criteria and constraints that would apply to this device, both from a science idea perspective and the perspective of someone who is going to use it?; I think that the device would have the criterion of being protective. The device would need to be comfortable. I would need to be able to move easily with the device on, so the device can’t come out from my knee too far. It needs to come on and off easily when I am sweaty. It would need to be affordable. I think the device can’t be too long. That might affect my ability to move my leg if it is too long. It needs to look cool and match the team colors. It definitely needs to be reusable and hold up to damage.. We have mentioned a lot of things that a designer would have to consider when creating this protective device. What might be a trade-off a designer of a knee pad would have to consider regarding the size of the pad and the safety of the player?; The bigger the size of the knee pad, the more potential for protection there is for the player, but the larger the knee pad, the less mobility or movement the player would have.. So you’re saying someone had to make a choice, or informed decision, about what feature was worth more in a design, even though size and safety are both important to the design?; Yeah, they had to look at the design and think about what would actually work when playing the game.. Do you see any other trade-offs that were made, and why do you think they made those trade-offs?; The design could have a much chunkier shape, but then it wouldn’t look as good, so they had to think about what people would buy versus what is safe. I don’t think people would buy it if it were chunky. They could have added more materials, but then maybe it would weigh too much for it to stay up on a kneecap area during the game. They had to limit the amount of materials used so the device is actually usable. I bet they could have used a better protective material on the inside around the kneecap, but when you are sweaty it may slide up or down. Keeping it in place is probably more important than a small boost in safety. (Accept all other trade-off-related responses.). Say, It seems that these trade-offs you are mentioning are very purposeful. It appears the designer really thought about the effects each change would have on the device and what would be important for safety but also user friendly at the same time. They had to make choices that they felt optimized the design but stayed within the limits of the device. Consider sources of feedback. Project slide H. Have students turn and talk with a partner about the prompts on the slide. - If we were to think about trade-offs for our devices, who would we need to consult to make sure we were making an informed decision about what was important? - Why would we need to consult and consider the feedback of others in the first place? Ask students to share their ideas about the first question with the class. Students should identify the following people: - the people who know the science behind the design - those who will use the design - those who will benefit from the design - those who are impacted by the design Ask students why we would need to consider feedback from others in the first place. Students should respond that without their feedback it may not meet their needs and people may not buy the design. Say, OK. We have done a great job learning about the science behind our designs, but if we are going to make our devices better, we need input from people who will use the designs, benefit from the designs, or be impacted by the designs. This group of people is referred to as stakeholders. We need to consult at least some of our stakeholders. Remind students that in the Tsunami Unit we learned about stakeholders. Stakeholders are the people who have interest in or concern for something. As we redesign our protective devices, our stakeholders will be the ones who would eventually use and/or purchase the devices. We could make the safest design in the world, but if our stakeholders would not purchase it or use it, then the design is useless. Stakeholder feedback will be needed and very important to the design of our protective devices. |  |

| ### 5 · HOME LEARNING | 8 min |
| --- | --- |
| MATERIALS: Redesigning Your Protective Device, Stakeholder Feedback Form |  |
| Prepare stakeholder feedback form. Explain to students that we have a form to use that will help us get feedback from stakeholders. The form is set up to ask the stakeholder questions based upon the design type and will help get targeted feedback to help optimize the design. Explain to students that sometimes when we think about giving feedback on designs, we limit our thinking to just critiquing a specific design and the choices made in that specific design instead of considering the possibilities of what could be created if a totally new, never-before-seen design was being made. To make sure we don’t limit our feedback, we will first need to get ideas from stakeholders about our potential designs without the stakeholders being influenced by our designs. Tell students that this can be done by either discussing designs that are already in use or by asking stakeholders generally what they would like to see in that kind of protective device design. This way students will be able to figure out what considerations are truly important to them without stakeholders feeling that they are being critical of the student’s design. After that, we can then introduce our design and get more-targeted feedback on our design and trade-offs. Distribute Stakeholder Feedback Form. Start by pointing out the two different parts to the handout. Part 1 is all about preparing and organizing our ideas to share with our stakeholders. Part 2 will be where we record the feedback from our stakeholders. Tell students that we will be going through part 1 together, and part 2 will be home learning. Introduce the decision tree. Project slide I. Explain that step 1 will help us know what to say to our stakeholders depending on our type of device. Walk through the decision tree in step 1, pointing out the two different columns on the slide that correspond with the columns on the handout. Say, Look at your design on Redesigning Your Protective Device. If you have designed a protective device that you think improves upon existing designs, like making an improved cell phone case, you will use the prompts in the first column. If you are creating a novel design for an object and there is no current protective device already available, use the prompts in column two. Let’s use some time right now to complete the column that applies to your design. Give students time to complete the column that is applicable to their design. Circulate and verify that students are using the correct column for their design. Explain step 2. Project slide J. Next, direct students to step 2 on their handout. Explain that the table will help students think about their design trade-offs and organize them so they can more-easily share their design decisions with their stakeholder. In the first column students will list the trade-offs used in their design. In the second column, students will give the rationale for the trade-off choices. Explain that the rationale is important to share with the stakeholder since a quick viewing of the device design may not allow the stakeholder time to consider the impact of trade-offs associated with certain design decisions. Go over the home learning with students. After explaining the table, move on to the home learning portion of the handout. Point out that step 3 is going to have students use the script identified in step 1 when discussing potential designs with their stakeholder. In part 2 students will get ideas from the stakeholders before sharing their design. Tell students that it is OK if they want to ask more than one stakeholder, and if they run out of room they can write on an additional piece of paper or in the margins of this home learning handout. #### ADDITIONAL GUIDANCE; This feedback will be important to obtain before the next class period. Due to dynamics outside of the school day, it may be important to provide some time during class for students to get feedback from others. If students finish steps 1-2 early during class, it may benefit them to start receiving feedback from another classmate. Remind students that feedback can come from any type of stakeholder (those impacted by the design, those that use the design, and those that benefit from the design). Other teachers, support staff, or administration can also be stakeholders for some devices. If students come to the next class period without the feedback completed, have students quickly get feedback from any available stakeholder.. On step 4, explain that students will use the trade-off table they will complete in step 2. They will share their decisions with their stakeholder and ask their stakeholder if they have any additional considerations or ideas. They will document those ideas in the table below step 4. Finally, step 5 is asking the stakeholder to do a review of the product. Tell students that it is OK to give the paper to the stakeholder for them to record their review. Students should make sure to let the stakeholder know that it is important to be honest so that the review can be used to optimize the design. Remind students that stakeholders are anyone impacted by the design, which means stakeholders can be other teachers, students, neighbors, friends, or even siblings or other family members. Allow students to work on the trade-offs table in step 2 until the end of the class period. As students are preparing to leave class, remind them to take home both Redesigning Your Protective Device and Stakeholder Feedback Form to get feedback from their stakeholders. They will collect feedback on their design from a stakeholder as their home learning, and the feedback will be used in class next time to consider design revisions✱. | #### ✱ ATTENDING TO EQUITY Universal Design for Learning: Engineers routinely engage stakeholders in the feedback process. As students work as engineers they will engage in these same processes to refine their designs. When students utilize the stakeholders in their communities to refine a design of their choosing, authentic engagement in the task increases as students see themselves, their decisions, and the effects that they can have on their communities. This will help students express solutions that are more relevant for their personal stakeholders and therefore ultimately more optimized for the problem they are trying to solve. |

### End of day 1

| ### 6 · SHARE FEEDBACK FROM HOME LEARNING | 5 min |
| --- | --- |
| MATERIALS: Redesigning Your Protective Device, Stakeholder Feedback Form |  |
| Draw parallels to market research. Project slide K. Explain to students that during their home learning they engaged in something real engineers do to get feedback from stakeholders. This is called market research. Companies and designers use different methods such as conducting focus groups, reviewing stakeholder thoughts on existing designs, analyzing stakeholder data from existing designs, and conducting large polls. But they also go and ask the consumers directly, just as students did during their home learning task. Explain that engineers also collaborate to create an optimal design, and the more stakeholder feedback that is received, the better the potential the design has to meet the needs of the stakeholders as a whole. Because of this, we are going to have an opportunity to share our feedback from our stakeholders with someone else in our class. Share stakeholder feedback. Project slide L. Tell students that they can use the prompts on slide L to share their feedback from stakeholders with their partner. If their partner shares some good feedback or a trade-off that they think is important, they should add it to their handout. Circulate as students are sharing the feedback they received from stakeholders on their devices. Listen to what trade-offs and feedback students were given by stakeholders and what trade-offs students would consider in the revision of their designs. Bring the class back together after the partners share to discuss the trade-offs and feedback shared by the stakeholders. Say, I heard a lot of different trade-offs that were mentioned by stakeholders, like ___, ____, and ____. It sounds like some of these trade-offs might be useful in our designs, while other pieces of feedback may not be as useful. Now that we have had a chance to talk to our stakeholders, let’s think about the things our stakeholders are prioritizing and determine if those changes would actually help to optimize the design. |  |

| ### 7 · UTILIZE A DECISION MATRIX | 15 min |
| --- | --- |
| MATERIALS: Redesigning Your Protective Device, Stakeholder Feedback Form, Decision Matrix, Additional Material Considerations Matrix |  |
| Say, Some of our trade-offs or suggestions mentioned by our stakeholders seemed to be practical, like it has to hold up to repeated uses or that it needs to be a certain size. Others, like how the design looks, should be considered too, but we may not think they are as important. We have to remember that to a stakeholder all of their feedback is important, so we need to consider all the feedback and trade-offs. When we think of potential stakeholder criteria such as durability, reusability, environmental friendliness, and protectiveness of our device, we are really evaluating the use of the materials that the design is constructed with. Explain to students that before engineers build their designs, they take into account the materials. Designing and redesigning prototypes can get expensive, so engineers try to get the majority of the issues resolved on paper before building the prototype. Tell students that in order to make sure we are using the right materials that align with the stakeholders’ feedback, we will get a chance to use a matrix to assess how well each material will potentially work for our designs✱. Distribute Decision Matrix. Project slide M. Pass out a copy of Decision Matrix to each student. Explain that students will be looking at the materials and considering how well they meet the stakeholder considerations and our science ideas. Students should remember using a matrix to evaluate choices in designs in the Cup Design Unit, Tsunami Unit, and Homemade Heater Unit. Tell students that we will add to our matrix just like we did in those units and focus on our material choices as we evaluate the trade-offs of each design. Tell students that just like in prior units, we will use the 1-to-5 scale to rate how well each material meets the considerations of the stakeholders, with 5 being the best and 1 being the worst. Begin by having students look back at their designs on Redesigning Your Protective Device and their feedback on Stakeholder Feedback Form and think about the different materials they are considering for their device. Point out the “Material choice” column on their blank matrix. Have students list these potential materials in that first column.Tell students that we will get to compare and evaluate these materials once we determine our important considerations that we want to use to rate these materials against. Say, We will be starting with a blank matrix since we all have different objects we are protecting. Part of the matrix will have to be completed on your own, but there are criteria that we can all agree upon. Let’s record that together. What are those criteria? Elicit responses from students. Students should say that one criterion is that the device needs to be made of materials that are good at protecting our objects. Ask students how we know that a material is protective. Students should respond that the material would reduce peak forces on the object it is protecting. Since students have already spent time exploring and naming the properties a material’s structure has to have to make it protective, explain to them that we will just use the criterion of force reduction on our matrix. Direct students add this to our matrix by writing “force reduction” in the red space under column head “Consideration #1”. Direct students to think about other considerations that they or their stakeholders thought were important. Ask students to share 2-3 more examples and show them on the slide where those considerations would be added to the matrix. Have students think about considerations specific for their designs and list them under the other ”Consideration” columns on their own handouts. Say, Now that we have thought a bit about what we want to evaluate for our material choices, we need to start assigning ratings to how well potential design materials meet our needs. Luckily, scientists and engineers have done many tests on the materials we have analyzed so far, and we were able to get some of that data to use in our redesign. Establish reliability of data. Project slide N. Explain to students that data for commonly used packaging materials were assembled for our use by the Michigan State University School of Packaging. This specific school at Michigan State University is focused on studying (and teaching) all aspects of packaging and packing materials. Researchers who work there took our list of common materials from Lesson 11 and ran the same amount of each material through a Life Cycle Analysis (LCA) program that is used to look at completed package designs. They were able to assess the environmental impacts of each material. The university also used some of their preexisting data on the same amounts of materials to rate each material in the other categories shown. Explain categories. Go over each category with students. Tell students that if they find a relevant category they should add it to their matrix as a new consideration. The ratings are given as a comparison from one material to another. Below is a description of each category: - Force reduction- how well the material reduces force on an object that it is protecting (The better the score is, the better the device does at reducing peak forces.) - Reusable- It can collide with other objects again and again and still be reused. - Robust- how well the device will work to protect heavy objects in a collision - Recyclable- how easy it is to recycle the material based upon availability of recycling sites for that material - Biodegradable- how well the material will biodegrade when thrown out - Low material cost- how cheap the material is for the same size sample - Low cost to work with the material in a design- how inexpensive it is to pay a worker to use that material to create a design solution - Water used for production- how much water goes into the creation of that material - Does not contribute to climate change- how small of an impact the creation of the material has on climate change Ask students to copy any relevant considerations that they would like to use from this list onto their matrix. Present data for student analysis. Project slide O. Distribute Additional Material Considerations Matrix to students. Show students the 5-point rating scale that has been used, which matches rating scales used in previous units. Remind students once again that to have a rating of 5 means that it works well with that consideration. A rating of 1 means that it does not work well with that consideration. Transfer data to the Decision Matrix. Project slide P. Tell students that in a moment they will transfer only the data that apply to their materials and design from Additional Material Considerations Matrix to Decision Matrix. Remind students that the ratings were done comparatively, so the materials were compared to each other to create those ratings. If students are going to consider materials not on Additional Material Considerations Matrix, have them think about how well the material would do compared to the others for each consideration and assign it a fair rating on the scale of 1-5. Tell students they can ask a partner to help verify any newly developed ratings as they are transferring the data to the Decision Matrix. Allow students time to transfer and develop ratings. | #### ✱ SUPPORTING STUDENTS IN DEVELOPING AND USING STRUCTURE AND FUNCTION In previous lessons, students have worked to uncover the properties of the small and microscopic structures of protective materials that aid in the reduction of peak forces on objects. In this lesson, students shift from thinking about the structure and function of the individual parts of the material to how well the material structure fits the design considerations based upon those properties for the use of the material within the overall system of our protective device design. |

| ### 8 · USE TRADE-OFF THINKING TO ANALYZE DESIGNS | 15 min |
| --- | --- |
| MATERIALS: Stakeholder Feedback Form, Decision Matrix, Additional Material Considerations Matrix, calculator (optional), 2 rulers (optional) |  |
| Establish a need for ranking criteria. Once students have had a chance to fill in the data on the matrix, bring them back together as a class. Project slide Q. Have students turn and talk with a partner about the question on the slide. - Use your “Total points” column to total up the ratings for your particular design. - If we were to look at the ratings and select the best material based upon which material has the highest overall rating, would that be the best material for the design? Why or why not? Say, OK, now we have all our important data and considerations rated together on one page. If we were to just go across the chart and look at the material with the highest total points, would that material be the best for the design? Why or why not? Review primary and secondary considerations. Allow students to respond. Students will mention that while some materials are rated lower in force reduction, the other considerations bring up the overall score. Remind students that in Tsunami Unit we determined that some considerations were more important than others. We reviewed our criteria and constraints and labeled the most important considerations as primary and the others as secondary. Ask students, If we were to establish a consideration across all materials that was the most important, what would it be? Students should respond that the primary goal of reducing peak forces on the object would be the primary consideration. Have all students put the letter “P” for primary above the ”Force reduction” column. Ask students if there are other considerations that are still important to the functionality of the device, but may not be a primary consideration. Allow students to give a couple of examples of these considerations, such as the consideration of reusability. Remind students that in the past we have used the secondary consideration designation for these criteria and constraints. Explain to students that we will label the consideration columns that fall into this category with the letter “S” as secondary considerations. Introduce tertiary considerations. Project slide R. Say, Since we are working with stakeholder input, all of the input is valuable. But, we also know that some input is more valuable than others. Some stakeholder considerations are based around the actual function of the device, and some may be superficial. Because of this, we will be adding a third level of classification called tertiary considerations. Explain that this third level of classification falls below the rank of secondary considerations. It will be the least important of our considerations when working with a material. Tell students that now that we have an established prioritization system, we will start assigning our considerations as primary, secondary, or tertiary. Determine primary criteria. Ask students to look at their charts on Decision Matrix and start by determining what should be considered primary criteria and label the space above these columns with the letter “P”. Remind students that the primary considerations are the most important items to the stakeholders. Determine secondary and tertiary criteria. After establishing primary considerations, students should look at the remaining considerations and determine if they are of more or less importance. If any considerations are deemed to be of a higher importance, but not quite primary criteria, students should label the space above these secondary criteria with the letter “S”. All other criteria should be labeled tertiary criteria with the letter “T” in the space above their corresponding columns. #### ASSESSMENT OPPORTUNITY; Building towards: 14.A Optimize the performance of a design by prioritizing the particular functions and properties of materials based upon stakeholder feedback to assess the relative effectiveness of the materials. What to look/listen for: Look for students to appropriately assign primary, secondary, and tertiary criteria based on stakeholder feedback from Decision Matrix. What to do - Establishing primary considerations: If students are struggling with determining if they should assign a primary label to a consideration, ask them if the stakeholder would consider not meeting that consideration a “deal breaker” when deciding if they should purchase the device. Also ask students if not having this consideration completely met would affect the device bad enough that it would become unusable. If the consideration is that important, it should be labeled as a primary consideration. - Establishing secondary considerations: Ask students if they have any considerations that seem very important to stakeholders, but the design would still function without meeting the consideration. If this is the case for any consideration, then it is labeled a secondary consideration. - Establishing tertiary considerations: Ask students if they have any considerations that might vary a lot in importance from one stakeholder to another or considerations that seem superficial. If any considerations fall into this category, students should label these as tertiary considerations.. Check criteria rankings with a partner. Display slide S. Ask students to share their labeling of primary, secondary, and tertiary criteria with a partner. Tell students that engineers also do this, and it is referred to as checking for interrater reliability. Students should give their partner feedback orally using the prompts on slide S. Students should adjust their rankings as needed based upon the feedback from their partner. Compare overall scores of materials. Alter point values based upon rankings. Discuss ways to elevate the primary criteria and constraints with students. Example prompts and responses are below. Suggested prompt; Sample student response. Let’s look back at our materials. We have ranked them as primary, secondary, and tertiary, but if we add up the total values, did the overall score change?; Mathematically, each material still has the same value.. Why could this be problematic?; We know certain criteria are worth more than others. We should have a way to show that they are worth more.. How do you think that we can alter the scale so that the items we think are the most important stand out?; Maybe we can make them worth more overall points.. OK. Engineers do this too using a process where they weigh different considerations based on importance. We can do that by weighting our scores. Let’s start with the most important stakeholder concerns. The primary considerations. How much do you think these should be worth?; They should be worth more than the others. Since we have primary, secondary, and tertiary, maybe they should be worth three times as much as the rest.. So since we have three levels, the primary considerations should be worth three times as much?; Yes. Yeah! That makes sense.. So what should the secondary concerns be worth?; They should be worth two times as much since they are secondary.. This means that the tertiary concerns can just stay the same value, since the primary and secondary are weighted more than them. Are we OK with that?; Yes.. Project slide T. Allow students time to do the multiplication involved with weighting the scores and then the addition in totalling up the new material score.✱✱ If some students struggle with how to do this multiplication, use the example on slide T to walk them through this process of weighting the criteria, multiplying based upon the rating, and then adding up the total new points for each material. Reflect on weighted scores. Project slide U. Have students compare pre vs. post weighted scores with a partner using the prompts on slide U. After students have had a chance to share, bring the class back together. Ask students if they believe these scores are more reliable. Some students may respond that these scores are more reflective of what the design needs to do, and some will be able to see a change in the highest rated material for their design. Others may point out that there are some criteria that should be “gating” criteria, that if they do not meet that criteria they should be automatically eliminated (they don’t make it through the gate to the next round of consideration). Verify for students that this process involves a lot of trade-offs from one aspect to another, and all criteria may not be met perfectly. But, by looking at the different considerations from a stakeholder’s point of view we learn how to make the designs work for the consumers. | #### ✱ SUPPORTING STUDENTS IN ENGAGING IN USING MATHEMATICS AND COMPUTATIONAL THINKING While students have had experience prioritizing criteria and constraints in the Tsunami Unit and Homemade Heater Unit, they had not quantitatively considered the impacts of the prioritization of these considerations. In this lesson, students prioritize stakeholder considerations and assign numerical rates to the different consideration priority levels in an effort to quantify the data. Multiplication is used to weight higher-ranking considerations against other, lower-ranking considerations. By comparing the new total scores for each material, students are able to get a picture of which material best meets the needs of the design. As students progress into the high school SEPs, they may modify this thinking with gating criteria, or criteria that must be met in order for the design to be evaluated further, for each material before it is evaluated quantitatively. #### ✱ ATTENDING TO EQUITY Some students may need assistance with the multiplication of a long set of numbers. Offer these students the use of a calculator that displays the multi-step equation as it is added to the calculator. Remind these students to use parenthesis where appropriate. Some students may benefit from the use of a ruler to keep track of numbers on a line or two rulers to block out unneeded data for the row and “chunk” only the needed information together for each material. |

| ### 9 · CONSIDER CONSEQUENCES OF TRADE-OFFS | 8 min |
| --- | --- |
| MATERIALS: science notebook, Redesigning Your Protective Device, Decision Matrix |  |
| Consider design optimization consequences. Say, Let’s think about some of these trade-offs we are now considering with the use of certain materials. It is clear from our different ratings that certain changes and decisions have unintended consequences. By elevating certain criteria, we may be discounting something else important to the stakeholder, or by only looking at the total scores we may miss something else that is more important to the overall rankings. Let’s spend some time reflecting on these choices that we would like to make to optimize our designs. Display slide V. Remind students that in the Homemade Heater Unit they worked with identifying what consequences their choices would have on the design and how that might affect stakeholders. Tell students they will consider the implications of trade-offs and prioritizing of considerations. Instruct students to create the chart on the slide on the next clean page in their notebook. Explain that in the first column students will write their different decisions regarding their materials and design, according to their most recent matrix exercise. In the second column, students will consider what affects these changes will have on other stakeholder concerns or input, and in the third column students will consider what affects this design decision would have on the overall design of the device. Give students time to complete this trade-off and prioritization reflection on their own. #### ASSESSMENT OPPORTUNITY; Building towards: 14. B Engage in an analysis to consider the trade-offs of prioritizing particular uses and functions of materials to assess their relative effectiveness for the optimization of the design. What to look/listen for: Students should list the design decisions they have made according to their redesign table and decision matrix and explain the potential effects (which can be positive and negative) that each design decision will specifically have on stakeholder concerns or considerations. Students also consider what effect the decision will have on the overall design of the device and explain how those decisions impact the performance of the design (which can be positive and negative). What to do - Column 1: If students need help listing their design decisions, direct them back to Redesigning Your Protective Device. Ask students what changes were made to the design and have them list those changes in column 1. Also have students look back at their decision matrix. Ask students what decisions they made to assign primary, secondary, and tertiary criteria. If they prioritized one criterion over another, have them add that design decision to column 1. - Column 2: Direct students to look at their list of design decisions in column 1. Ask students, How would this decision affect the people who use or rely on this device? Would those people have any additional concerns because of this choice? Ask students these two questions about each design decision listed. - Column 3: Direct students to look at column 1 one last time and think about how these decisions impacted the actual physical structure of the device. Ask students what would happen to the device if they made a different decision, used a different material, or prioritized something else instead of what they had chosen. Tell students to write those potential outcomes of different decisions on their device structures in this third column.. |  |

| ### 10 · NAVIGATION | 2 min |
| --- | --- |
| MATERIALS: None |  |
| Say, We have done a lot of work to ensure we are making informed choices for our designs. Now that we have some great ideas for how to improve our designs for our stakeholders, let’s take some time at the start of our next lesson to synthesize all the data and trade-offs. We can then craft an argument to support our design for marketing purposes. We will also use our science ideas to evaluate some new protective designs! |  |